# Working Principle of LED

1.	Briefly describe the working principle of LED when it is connected in forward bias (applied voltage = 1.2 V).

2.	By observing the motions of the electrons and the holes, explain why a LED does not work when it is connected in reverse bias (applied voltage = −1.2 V).

# Working Principle of LED (Answers)

1.	Briefly describe the working principle of LED when it is connected in forward bias (applied voltage = 1.2 V).

When the LED is connected in forward bias, an E-field pointing from the p-type semiconductor to n-type semiconductor is set up between the teminals of the LED. The electrons inside the n-type semiconductor move towards the positive terminal while the holes inside the p-type semiconductor move towards the negative terminal. When the electrons jump across a p-n junction, they meet the holes and release energy as visible light.

2.	By observing the motions of the electrons and the holes, explain why a LED does not work when it is connected in reverse bias (applied voltage = −1.2 V).

If the LED is connected in reverse bias, the E-field between the terminals is reversed. The electrons are attracted by the positive terminal while the holes are attracted by the negative terminal. The electrons and the holes move away from the p-n junction and thus the LED does not light up.